VILNIUS UNIVERSITY
SIAULIAI ACADEMY

PROGRAMŲ SISTEMOS BACHELOR STUDY PROGRAMME

Software engineering

ANNA KUTOVA

Programming of Embedded Systems

Laboratory work No.1

I/O Ports

Šiauliai, 2025

### Laboratory Work Report

#### 1. The Aim of the Laboratory Work

The aim of this laboratory work is to develop a MicroPython program for the NUCLEO_F756ZG microcontroller that processes input signals from buttons and performs actions based on state changes. The program should detect transitions between logical levels (1 to 0 and 0 to 1) and execute specific operations accordingly.

#### 2. Variant No and Data

Variant No: 9

Date: 13/02/2025

#### 3. Program Algorithm

#### 4. Program Body with Comments

from machine import Pin

import time

import pyb

# Initialise input pins

pin_D2 = Pin("D2", Pin.IN, Pin.PULL_UP)  # Button with pull-up resistor

pin_D3 = Pin("D3", Pin.IN, Pin.PULL_DOWN)  # Button with pull-down resistor

x = 100  # Initial value

state = "INITIAL"

# Store previous states of the buttons

prev_D2 = pin_D2.value()

prev_D3 = pin_D3.value()

while True:

# Read current states

current_D2 = pin_D2.value()

current_D3 = pin_D3.value()

# Detect changes in D2

if current_D2 != prev_D2:

if current_D2 == 0:

state = "BUTTON PRESSED"

pyb.LED(1).toggle()  # Toggle LED when button is pressed

else:

state = "BUTTON RELEASED"

print(f"State: {state}, x: {x}")

# Detect changes in D3

if current_D3 != prev_D3:

if current_D3 == 0:

x -= 2  # Decrease x only on state change

print(f"State: {state}, x: {x}")

# Update previous states

prev_D2 = current_D2

prev_D3 = current_D3

# Small delay to reduce signal bouncing

time.sleep_ms(50)

5. Conclusions

In this laboratory work, a MicroPython program was successfully developed for the NUCLEO_F756ZG microcontroller to process button inputs based on state changes. The program correctly detects transitions between logical levels and performs actions accordingly. This approach prevents continuous execution of the same action while a button is held down. The use of a small delay helps minimize signal bouncing. The experiment demonstrated how to work with GPIO, implement state tracking, and toggle LED outputs.